Camera Calibration Tool

Presentation:

This program is a Camera Calibration tool used to extract parameters (intrinsics, extrinsics, and distortion coefficients) necessary for computer vision calculations. It uses the Aruco module based on the openCV library.

Manual:

The tool enables you to perform a calibration with a live video feed from selected cameras.

Please follow the instructions below for detailed steps:

- Print the Aruco Board provided in the program folder(ArucoBoard.png), Size: 30x42 (A3 Format). Stick it on a hard cardboard: the paper must remains as flat as possible during the calibration

- Place the camera on a stable mount, select it in the drop menu (“Select Camera”) and make sure it is in focus. Focus mode should be Manual and not Automatic.

- Hold the board in front of the camera until all markers are detected,  then click on the "Add Image" Button, or press the “A” Key.

- Take at least 10 images of you calibration board in different positions and orientations without moving the camera between each snapshot. See examples below:

- Click the "Calibrate" button to generate the calibration parameters from your image list.

Notes

- The reprojection error is displayed to score the calibration precision: it should be close as close to 0 px as possible.

- Make sure you use good  lightning conditions during the calibration process. Avoid reflections and lens flare.

- Reset the image list if you want to start over with the "Reset" button.

Screenshot

Camera Parameters:

Once the calibration is finished, a folder will pop up containing the camera parameters file. It consists of an XML document with the format shown below:

Resources

OPENCV  	https://docs.opencv.org/trunk/d5/dae/tutorial_aruco_detection.html

https://docs.opencv.org/3.4/da/d13/tutorial_aruco_calibration.html

ArucoUnity 	https://normanderwan.github.io/ArucoUnity/manual/calibrate-a-camera.html

Calibration Tips	https://stackoverflow.com/questions/12794876/how-to-verify-the-correctness-of-calibration-of-a-webca